AN - 2 TP SP-ZET Basic aircraft data Type / model AN-2 P / SP-ZET series 174 Year manufactured 1977 Manufacturer WSK - PZL Mielec, Polska Factory serial number 1 G 17460 Register number 4062 Certificate of Airworthiness valid through October 14, 2010 PZU SA TPL and Aerocasco comp. insurance valid through October 26, 2010 Total flight time since last overhaul / Last overhaul date - airframe 241 hours 37 min / August 2006 - engine 245 hours 31 min / March, 2006 - propeller 245 hours 31 min / December 05, 2007 Total flight time since beginning of operation 2845 hours 56 min Operating time to overhaul 654 hours 04 min or till 2016

Operating data Crew 2 Number of passenger seats 6 Cargo area up to 200 kg

Technical data Dimensions wingspan: 18.18 m length: 12.40m height: 4.10 m lift area: 71.50 m2 Weight empty weight: 3450 kg usable payload: 2050 kg takeoff weight: 5500 kg fuel tanks: 1200 l fuel consumption: approx. 160 l/h Specifications maximum speed: 258 km/h cruising speed: 185 km/h minimum speed: 90 km/h climbing speed: 3.5 m/s maximum altitude: 4400 m range: 900 km takeoff roll: 170 m landing roll: 215 m Basic engine specifications Engine type, series ASz-62 IR, series 16 Year manufactured 1990 (engine installed an aircraft) Factory zero number K 1651008 I main overhaul WSK PZL Kalisz May 31, 2005 Total flight time since last overhaul / Last overhaul date - engine 215 hours 31 min / March, 2006 Time to main overhaul 784 hours 29 min. of operation. Basic propeller specifications Propeller type AW-2 Manufacturing date June 30, 1975 (propeller installed an aircraft) Factory zero number W 195150 I main overhaul January 08, 2002 usage after overhaul 1500 hours or 6 years Total flight time since last overhaul / Last overhaul date - propeller 245 hours 31 min / December 05, 2007 Calendar usage valid through December 04, 2013 Remaining to main overhaul 1254 hours 29 min. of operation. Radio-navigation equipment (installed an aircraft) Transceiver SL-40 UNIT “Garmin” No. 25802519 Transceiver GNS 430 “Garmin” No. 97135792 GA-56 “Garmin” Cross indicator No. 59422365 Transponder “Garmin” GTX 327. Additional, nonstandard equipment: Air-conditioning with remote control Sony audio system Noise muffler Drinkable and usable water tanks built into the aircraft structure Leather armchairs WC. Description Lounge version of biplane, in very good technical and overall condition of passenger section equipment. It has a lounge with armchairs and couches made of brown leather, a wooden table (dark brown), drink-bar, kitchen area with a refrigerator. WC section. In the cockpit: brown leather seats. Carpeting - beige. The entirety constitutes a very attractive and comfortable interior. Additional equipment: air conditioning and noise muffler increases comfort during flight.. Location Poland. Contact Bernard Kwapiński telephone: +48 609 026 370 e-mail: bk@ekopark.pl.