ANTONOV AN-2

1a

Check-list

Normal procedures

Unusual procedures

Emergency procedures

- Figures, numbers & facts

# N82AN

Courtesy  of  Dr. Phil Clarke

& Revised by John Webster

1b

- PREFLIGHT INSPECTION:

2a

- 1. Cockpit check:

- Control lock............................…....removed & stowed

- Parking brake...............................................................SET

- Log book, flight doc. & legal forms...................on board

- All switches..................................................................OFF

- Magneto switch.................................................." 0 " (OFF)

- Windows.................................................................CLEAN

- 2. Engine group:

- 1.   Propeller..CLEAN, turned CCW 15 - 20 times

- 2.   Engine cowling latches....…........ALL CLOSED & LOCKED

- 3.   Oil cooler................................................................cleaN

- 4.   Dust filter (if installed)..........................check position

- 5.   Oil purge............(bottle stowed)..............................DONE

- 6.   Oil quantity.................................CHECKED (50 - 70 litres)

- 3. Main landing gear:

- Tires condition & pressure (40 PSI )………...……..........OK

- Brakes....................................................................CLEAN

- Check shock absorber compression according to scale:

- 90  9 mm when airplane weight is 3600 Kg	  7920 lbs

- 117  17         "        "             "     "   4200 Kg	  9240 lbs

- 135  14         "        "             "     "   4800 Kg	10,560 lbs

- 148 15          "        "             "     "   5250 Kg	11,550 lbs

- 154  15         "        "             "     "   5500 Kg 	12,100 lbs

- 4. Left & right wings:

- Fabric skin.................................................no defects

- Tie downs & Control locks...............................REMOVED

- Carrying tapes holding & tighting...........not too weak

- Lower surface of upper wing.........no gasoline leaks

- Pitot cover.......................................................removed

- OAT probe cover.............................................removed

- Navigation & landing lights glasses....….................clean

- Side fuselage.......….no cracks, no deformations

- 5. Tail section:

- 1.   Shock absorber compression......normal... (50-70 mm)

- 2.   Shock absorber.........................................no oil leaks

- 3.   Tire pressure......................................................normal

- 4.   Fabric skin of control surfaces....................no defects

- Navigation light...........................................undamaged

- Tie downs & Control locks...............................REMOVED

- 7.   Fairing mounting screws..........tight & none missing

- 6. Pax - Cargo cabin:

- Cargo loading, arrangement & mounting...............…...OK

- Rear section loading...................................................OK

- Doors.............................................................SECURed

- All "remove before flight" flags..removed & count=8

PREFLIGHT

2b

- BEFORE STARTING:

- Propeller…(within 10 minutes) Pulled CCW  15 - 20 times

- Oil collector can.....................................REMOVED & STOWED

- Doorstep..............................................REMOVED & secured

- Emergency manhole................................closed & secured

- Magneto switch.........................................................." 0 " (OFF)

- Flight controls..............................freedom of movements

- Air system charging valve.................................................OPEN

- Air pressure.......................................................... Check > 30

- Parking brake......................................................................SET

- Brake pressure system............................................check > 6

- Battery switch......................................................................ON

- Bus voltage.................................................................... > 24 V

On control desk, switch ON:

10

- fuel tank meter

- engine unit gages

- flap position & oil cooler shutters indicators

- OAT + mixt. T°

- oil cooler shutters

- cowl flaps

- fire ext.

- warning bell  f

- Rotating Beacon

- Left & Right fuel gages..............................check quantity

- Trim tabs........set to neutral....... (3 GREEN INDICATORS LIT)

- Oil coolers shutters & cowl flaps............check deflection

- Fuel selector valve......................................................" both "

- Fire Extinguisher....................Checked (yellow indicator ON)

- "Chip Monitoring" button..............................press to test

- ENGINE START: if CHT & Oil T° > 05° (if not: oil dilution)

- Propeller…(within 10 minutes) Pulled CCW  15 - 20 times

- Propeller control...      .........................FORWARD (Low Pitch)

- Mixture control...   ..............................BACKWARD (Full Rich)

- Fuel master lever.....................................OPEN = FORWARD

- Carburetor heater..   ..........................................................OFF

- Oil cooler shutter........................................................CLOSED

- Cowl flaps...................................................................CLOSED

- Fuel pressure (with manual pump)...........................0.25 - 0.35

- Primer pump to cylinders.........if CHT < 80° :

- 2 - 6 injections MAX in the summer (or soon after stop)

- 6 - 10 injections MAX in  the winter

- Primer pump lever.......................................................LOCKED

BEFORE STARTING - START

MAKE SURE PROP. AREA IS....................................... CLEAR

3a

- Magneto switch..............................................................OFF " 0 "

- " Starting seqence " switch.....................................................ON

- Starter....................ON.........................Until 8 ON AMPMETER

- Then toggle the switch to interconnect engine shaft

- Magneto switch........................................................BOTH " 1+2 "

- After start: fuel pressure with manual pump..................025 - 0.35

- Rem: if engine fires back into carburetor, do a few smooth

- injections with the primer pump, without touching the throttle

- Throttle (when engine runs smoothly)...................700 - 800 RPM

- Check oil pressure..... Normal (> 3).....within 10 sec, if not: STOP

- "Starting sequence" switch....................................................OFF

- Generator...............................................................................ON

- A/C Converter.........................................................................ON

- Primer pump lever........................................................LOCKED

- ENGINE WARM UP:

- Throttle 1000 RPM until.... Oil T° rises, but in less than 3 minutes

- Throttle 1200 RPM until..........................CHT >100° & Oil T° >30°

- Throttle 1600 RPM until..........................CHT >120° & Oil T° >50°

- ENGINE TEST:

- - to be done in 4 minutes MAX (insufficient cooling)

- - control wheel in backward position all-through the test

- Establish max. continuous rated power...........................2100 / 9

- Check "in the green":

- Oil pressure..............................4 - 5

- Fuel pressure..................0.25 - 0.35

- Oil T°..................................60° - 70°

- CHT.......................................< 215°

- Throttle. .................................................................... 2000 RPM

- Magnetos check (15 sec. each)....................... drop < 60 RPM

- Throttle.... ...................................................................1900 RPM

- Propeller.. ............"coarse pitch".........result = 1450 - 1550 RPM

- Propeller.... .............."fine pitch".........result = 1900 RPM by itself

- Rem: repeat 2 or 3 times in cold weather

- Throttle. ..................................................................... 2100 RPM

- Set with propeller gov..       1900 RPM & then with throttle 10-15:

- engine speed as a result must remain constant

- Throttle. ................900 RPM................then swiftly back to 1900:

- Rem: engine may increase to 2000 - 2050 but stabilize

- to 1900 in 3-5 sec.

WARM UP  -  TEST

Throttle.. .................................................................1850 RPM :

3b

- A/C voltage................................... > 75.5 V

- mixture check     : 1-2 cm from extreme forward position

- carburetor heater      check (10-15 sec): 150 - 250 RPM

- drop. Warning: backfire should not occur this moment

- Set take-off power for 10 - 15 sec, must have:

- Engine speed: 2150 - 2200 RPM

- M.P: 10.5 (if more, do not set max throttle for take-off)

- Set throttle to....................................................Idle (< 20sec) :

- Engine speed.................550 RPM

- Oil pressure............................ 1.5

- Fuel pressure........................ 0.15

Oil T°.............................. 60° - 75°

Gen warning

- Generator OFF: check                         ON & note the

- negative deviation (to the left) of the ampmeter pointer

Generator: ON

Gen warning

- Add throttle:                           should go OFF at 900-1100 RPM

- 2 or 3 engine transitions...... Idle <-> 2100 RPM (in 2 - 3 sec),

- CHT must be > 120°

- BEFORE TAXI:

- Pax & cargo doors.....................................CLOSED & Locked

- All necessary switches..........................................................ON

- Flaps upper & lower................................................operative

- Engine control lever positions. ...... ...... ....................correct

- Indications of engine gages..............normal "All in the green"

- Fuel quantity in each tank..........................................checked

- A/C Converter.........................................................operative

- "Artificial horizon, course indic. & gyro compass"  switches..ON

- Gyro instruments..................................….....................SLAVED

- Navigation instruments..........................….....checked & set

- GPS....................................................................................ON

- Transponder.....(1200)...." STDBY "......& 2 switches….......ON

- Air pressure in pneumatic system.................................... > 40

- Wheel chocks.............................................check removed

- Tail wheel ..............................................................UNLOCKED

- Block time..............................................................RECORDED

Note : Tail wheel locking has been disabled on this aircraft.

BEFORE TAXI

TAXI :

4a

- 1. Lights...............................................................As required

- 2. If snow or OAT................................................................... < 0°

- pitot heat........................................................…...ON

- keep mixture T° > 8° with carburetor heater... .......ON

- 3. All necessary switches.........................................................ON

- 4. Brakes......................................................................checked

- In turns, gyro indications.........................................CORRECT

- ROTATING BEACON ………………………………..………ON

- BEFORE TAKE-OFF:

- Voltage & Load meter...................................................check

- Navigation instruments & Com's ......................confirm set

- Carburetor heater.....  .......................................................OFF

- Mixture.....   ..........................................................FULL RICH

- Fuel selector valve....................................................." BOTH  "

- Altimeters.........................................................................SET

- Gyro compass.........SLAVED & SET TO RUNWAY HEADING

- Set trim tabs:

- Elevator........DOWN: If heavy for 10 sec.; if not 3 - 5 sec.

- Rudder..........RIGHT for 3 seconds

- Flaps..................................set for take-off (usually 15°)

- Flight controls............................Freedom of movements

- Artificial horizon.......................check normal indication

- Transponder......................................................set on "Alt "

- Engine gages.........normal indications: " all in the green "

- Cowl flaps & Oil cooler shutters.....................................OPEN

- Lights..............................................................AS REQUIRED

- Pax seat belts............................................…........FASTENED

- Take-off time............................…........................RECORDED

- Parking brake.............................….......................Released

TAXI - BEFORE TAKE OFF

- TAKE-OFF:

4b

- 1. Without flaps:

- Engine parameters..........................9 / 2100

- Speed up to...................................140 Km/h  75 Kts

- then resume climb

- 2. With flaps (wind < 20 Kts): reduces take-off distance by 35 %

- If 15°: engine...................................9 / 2100

- If 25°: engine...................................9 / 2100

- If 30° (max weight)......................10.5 / 2200

- rotation speed...........................85 - 90 Km/h  48 Kts

- Speed-up to ..................................120 Km/h  65 Kts

- Flaps over 150 Ft..............gradually drawn-in

- Full drawing of flaps at...................140 Km/h  75 Kts

- then resume climb

- CLIMB:

- Establish initially.............................9 / 2050 (9 / 2100  if urgent)

- When altitude > 150 Ft.............................................8.5 / 1850

- Speed..................................................140 - 150 Km/h   78 Kts

- Rem: if max. rate of climb required, keep 5° of flaps

- until passing 1500 Ft before retracting.

- Check :

- Oil T°...................... < 70°

- CHT ......................< 215°

- Lights................................................................considered

- CRUISE:

- Engine parameters (RPM - M. P.)...................Check table

- Check :

- Oil T°........................................ 60° - 75°

- CHT: 120° - 215° (recommended: 150° - 215°)

- Oil Pressure.................................... 4 - 5

- Fuel Pressure........................ 0.25 - 0.35

- Compressor pressure...................45 - 50

- Voltages.......................> 28.5 V & > 75 V

- Check mixture T° for carburetor heater      need.......if < 0°an..    .....................................As required (Max cht: 215°)

- Flaps................................................................................. 0° ?

TAKE OFF - CLIMB - CRUISE

- DESCENT :

- 5aaaaaaaa	DESCENT:

- Establish................................ 5.2 / 1500 (= 180 Km/h  97 Kts)

- Maximum speed for descent:

- Quiet air.......................220 Km/h	119 Kts

- Turbulent air.................190 Km/h	102 Kts

- Maintain ..........................................CHT > 120° & Oil T° > 50°

- Rem: if necessary close cowl flaps & oil cooler or descent by steps

- Carburetor heater      ON If.................................... OAT < -15°

- Fuel selector.............................................................." BOTH "

- Parking break...CONFIRM OFF & PRESSURE IN BRAKES "0"

- Air pressure ............................................................check > 40

- Flaps........................................................RESYNCHRONISED

- Flaps……………………………………………….….as required

- FINAL:

- Throttle.. ............................................................................< 5

- Propeller.... .........................................................Full pitCH

- Speed.........................................................< 140 Km/h 75 Kts

- Flaps....................................CHOOSE SETTING (usually 15°)

- Landing lights......................................................................ON

- Tail wheel locking..............................................considered

- CRoss wind……………..……………………..considered

- CLEAR OF RUNWAY:

- Tail wheel.............................................................unlocked

- Landing lights...................................................................OFF

- Taxi light..........................................................CONSIDERED

- Pitot heat..........................................................................OFF

- Flaps....................................................................................0°

- Transponder.............................................................." STBY "

- Cowl flaps & Oil cooler shutters....................................OPEN

DESCENT - FINAL - CLR

5b

- COOLING DOWN & SHUT DOWN:

- Avionics & XPDR switches.................................................OFF

- Set throttle.........................................................700 - 800 RPM

- Cowl flaps & Oil cooler shutters.......................................OPEN

- Let CHT cool down.......................................................< 120°

- Electrical switches & radios.................................................OFF

- For a few seconds: throttle........1700 RPM, then.........800 RPM

- Fuel master valve.   .....................................................CLOSED

- Throttle...PROGRESSIVELY INCREASED DURING SHUT-OFF

- Cowl flaps & Oil cooler shutters......if T° < 100°...........CLOSED

- Air system charging valve............................................CLOSED

- Controls & ailerons......................................................LOCKED

- Oil & Air system purges...................................................DONE

- If parked outside....................AIRCRAFT MUST BE TIED-DOWN

- And Control Locks installed.

- Flight Plan Closed ?

COOLING DOWN & SHUTDOWN

6a

ENGINE TEST GRAPHIC

- ACCEPTABLE AIRSPEEDS (Km/h)		(Kts)

- In turbulent air:                  < 190 		102

- Diving (2200 RPM) = VNE = 300 	160

- To extend flaps:                 < 150 	 85

- With 30° flaps:                   < 150 	 85

- With 40° flaps:                   < 130 	 70

- Minimum in horizontal flight: 120 		 65

- WIND LIMITATIONS (in Kts):

- Normal		Slippery

- Take-off & Landings:		airstrip		 airstrip

- Head wind			   32 		    16

- 45° Cross wind		   14		     8

- 90° Cross wind, no flaps	   14		     6

- 90° Cross wind, flaps	    8		     6

- Taxiing:			   32		    16

- AIRSPEED INDICATOR (Km/h)	(Kts)

- VNE:		          300	160

- Yellow zone:    225 – 300	121-160

- Green arc:       100 – 225	54 -121

- Flaps white arc:  100 – 130	54-70

- RATE OF CLIMB at Max. Landing Weight (5250 Kg)

- With 8.5 / 1850 @ 140 Km/h   75 Kts:

- FL 0   to FL 65     :    600 Ft/min

- FL 65 to FL 90     : > 400 Ft/min

- FL 90 to FL 120   : > 200 Ft/min

- FL 120 to FL 150 : > 100 Ft/min

- Rem: service ceiling = 4400 metres at 5250 Kg

- =14,400 ft. at 11,550 lbs.

6b

- FUEL CONSUMPTION (From T.O. to LDG):

- FUEL SEQUENCE:

- T.O. & Climb: Both

- Before leveling: RIGHT for 120 - 150 litres

- then: LEFT for 120 - 150 litres

- then: RIGHT for 120 - 150 litres...

- When total remaining fuel is about 300 litres: BOTH

- Notes:

- Fuel alarm = less than 110 litres remain  (55 litres in each tank)

- Continuous use of BOTH: more fuel used from the LEFT tank

7

PAYLOAD - CARGO vs FUEL - DATA

- CARGO WEIGHT versus FUEL QUANTITY IN TANKS:

- It is possible to take-off with a crew of two + 75 litres of oil taking ... Kg of cargo, with …litres of fuel on board

- 1000=2200 lbs	1200=317 gal (max. fuel)

- 1150=2530 lbs	1000=264 gal

- 1300=2860 lbs	  800=211 gal

- 1450=3190 lbs	  600=160 gal

- 1600=3520 lbs	  400=106 gal

- 1750=3850 lbs	  200 =53 gal (min. T.O. fuel)

- MAXimum PAYLOAD CALCULATION:

- 2120  Kg	4660 lbs

- ... litres of fuel X 0.72 = -            Kg

- ... litres of oil   X  1     = -            Kg

- ... crew            X 80    = -            Kg

- PAYLOAD    =             Kg

- ENGINE & ACCESSORY UNITS DATA:

- CHT.....................................150 - 215°, mini. OAT for starting +5°

- Oil T°.......................................60 - 75°,               "

- Fuel Pressure:

- at power..................0.25 - 0.35

- at idle................................0.15

- Oil pressure..................................4 - 5

- Starter:

- max consecutive startings........................................4

- min breaks between startings sequences.........20 sec

- time of switching ON the electric motor............13 sec

- time of turning it manually.............................5 - 6 min

- Compressor:

- pressure...........................45 –50 Kg/sq.cm

- max temp................................110 °

- time of max filling from 0........11 min

- Converter:

- voltage..................................28.5 V

- loading current.......................100 A

- max loading........150 A in max 2 min

7b

- ENGINE MANUAL START (Low batteries, no G.P.U.):

- Propeller control.    ................FORWARD (Low –fine pitch)

- Mixture control....... .................................FORWARD (Full rich)

- Throttle... .........................................700 - 800 RPM Position

- Fuel master lever.. ..............................................FORWARD

- Carburetor heater... ........................................................OFF

- Oil cooler shutters.......................................................CLOSED

- Cowl flaps...................................................................CLOSED

- Magnetos..........................................................................OFF

- Manually turn the prop.............................5 - 6 revolutions

- Fuel Pressure (with manual pump)...........................0.25 - 0.35

- Primer pump to cylinders:

- 2 - 3 injections MAX in the summer

- 5 - 8 injections MAX in the winter

- Primer pump lever..............................................................OFF

- Battery................................................................................ON

- Manual crank:

- inserted  into the special sleeve behind the copilot

- turned clockwise gradually increasing speed 'till 80 RPM

- Starter.................................................................................ON

- Magnetos...................................................(1+2)............" BOTH "

- Check oil pressure....Normal (>3)...within 10 sec, IF NOT: STOP

- " Starter sequence " switch............................................."OFF "

- Generator.............................................................................ON

- Primer pump lever.......................................................LOCKED

- Note : Most An-2 aircraft in U.S. have had the manual starting Crank disabled.

8a

- AT ENGINE START IF FUEL BURNS WHEN CARBURETOR BACKFIRES:

- Magnetos........................................................" OFF "

- Starter control.......................................................ON

- Rem: Do not touch throttle at this time

- 3. If flames do not go out: "FIRE EXTING."...............ON

- ENGINE FIRE:

- Fuel master lever........ ..............................CLOSED

- Fuel selector.................................................CLOSED

- Magnetos & Converter...................................." OFF "

- Cowl flaps.....................................................closed

- " FIRE EXTING. " button............................PRESSED

CARB. BACKFIRE -ENGINE FIRE

8b

AIR START:

 Cure the problem first:

- Fuel Selector………………………………… BOTH

- Low fuel pressure (<0.25).............MANUAL PUMP

- Fuel shut-off valve...............................FORWARD

- Too much lean.....................................FULL RICH

- Carb. icing (mixture T° low)....CARB. HEATER ON

-  Air start: Rem: propeller will always keep turning

- Set speed...........……....150 - 160 Km/h   80-85 Kts

- Throtlle......................................................800 RPM

- Magnetos....................................... ‘’2-1’’..." BOTH "

- Fuel tanks.................................................." BOTH "

- Fuel Pressure with manual pump.............0.25 - 0.35

- Mixture...................................................FULL RICH

- Engine should restart at this point

- Rem: if engine has been stopped for a long time, use the starter for a few seconds.

- GLIDING:

- Engine....................................................STOPPED

- Propeller.............................................FEATHERED

- Any turn .......................= 60 metres loss in altitude

- FLY THE  AIRPLANE

AIR START - GLIDING

100 m

800 m

130 Km/h - 70 Kts

9a