Magneto replacement and timing for the ASZ-621R  Engine.

Read the magneto automatic advance angle value stamped on the rear cover

of the magneto.     (see /pcs. 7, Fig. 2.2/,)

_________________________________________________________

Magneto automatic       	Magneto advance angles during

advance angle           		the first cylinder compression

(value stamped          		stroke before TDC (in crank-

on the magneto          		shaft revolution degrees)

cover)

RH magneto        LH magneto                                                   				(front spark-         (rear spark-                                                  				     plugs)                plugs)

__________________________________________________________

13.0                             8.6                            3.5

13.5             		8.0                            3.0

14.0                             7.5                            2.5

14.5                             7.0		         2.0

15.0   			6.5                            1.5

15.5                             6.0                            1.0

16.0                             6.0                            1.0

16.5			5.0		         0.5

17.0			5.0		         0.0

17.5			4.5		        -0.5 (after TDC)

18.0			4.0		        -1.0	""

___________________________________________________________

BSM-9F MAGNETO REPLACEMENT

Remove number 1 front spark plug, rotate prop until number one cylinder is at TDC

compression. Remove sight plug on left side of engine nose casing. (Degree marks

for the timing of magnetos can be seen through that hole.)

When installing these magnetos, set the prop reduction drive gear, (pictured on top page 1),

on magneto ignition advance angle.

20.0 degrees before top dead center for right magneto.(rear plugs)

15.0 degrees before top dead center lor left magneto.(front plugs)

Note: This information is found in Service instructions No, WT-62.04.01K,

Section No. 2.8, page 2-13.

-----------------------------------------------------------------------------------

After finding the automatic degree value stamped on the rear of the magneto case,

go to the chart (above) and find the corresponding degree number at which to set the

prop reduction drive gear.

Align the magneto rotor with the mark on the rear magneto case.

Install the magneto on the engine so that the points are just breaking when the

reduction gear is aligned with the proper degree mark in the chart (above).

If the main finger on the rotor does not align with the mark on the rear case after

setting the break, loosen the three screws on the rotor and align the rotor.

When the magneto is installed, try to keep it somewhat centered in the mounting slots

to allow some adjustment for future re-timing.

----------------------------------------------------------------------------------------