Valve setting Procedure for ASZ-621R engine

(1) Turn prop until No. 1 cylinder up on top dead center of compression stroke.

(2) Remove inspection plug on left side of nose.

(3) Find No. 1 cylinder mark on reduction gear.

(4) Align mark with slot in plug hole.

(5) Set intake and exhaust valve clearances at 0.015.

When making this Setting, check rocker roller for very slight movement: if roller will not move, it is set too tight; if roller is too free, it is set too loose. Be sure not to align mark on adjusting screw with slot in rocker arms on cylinders No.1, 2, 3. 8, or 9 only.

The mark an adjusting screw on rocker arms in cylinders No. 4, 5, 6, and 7, does not matter.

(6) After all valves have been set at 0.015, turn engine through two revolutions, aligning No. 1 cylinder on top dead center compression strike and aligning mark on reduction gear with mark in inspection plug hole.

(7) Check valve setting with feeler gauge or 0.017 and 0.012 as a nogo and a go. If valve adjust is loose on 0.017,  tighten till 0.017 is snug  tight on 0.012, loosen till it is snug on 0.012 .

Note: rocker is not too tight on feeler gauge when rocker roller will turn with a little drag. When screw adjustment is made and mark on screw aligns with slot in rocker arm, it is better to be adjusted on loose side than on tightside.

(8) Continue setting of valves with nogo and go settings until all valve settings fall into this range

(0.017 to O.012).